Debugging the stale pppoe sessions at the pfe.

- labroot@newton-re0> show subscribers user-name sharavnew extensive

Type: PPPoE

User Name: sharavnew

IP Address: 40.40.0.11

IP Netmask: 255.255.255.255

Logical System: default

Routing Instance: subscriberRI

Interface: pp0.3221235526

Interface type: Dynamic

Underlying Interface: ge-1/0/1.3221235525

Dynamic Profile Name: profile_ABC1

MAC Address: 00:11:01:00:00:01

State: Active

Radius Accounting ID: 20083

Session ID: 20083

PFE Flow ID: 10156

VLAN Id: 160

Login Time: 2021-04-14 22:00:52 PDT

IP Address Pool: poolRI

Accounting interval: 600

{master}

labroot@newton-re0>

- labroot@newton-re0> start shell

Important Note: use the fpc number on which the problematic subscriber is active. Based on the subscriber extensive output that you shared, it is on ae4 which uses xe-8/x/x i.e fpc8. However, please verify this once

%

% cprod -A fpc1 -c "show vbf flow" | grep "10156”

10156  PPPOE        1073751809       .pp.9985     uflow:10154  session=1

16797273  IP ROUTE                  5:40.40.0.11     uflow:10156

%

- % cprod -A fpc1 -c "show jnh inline-ka session 0 ppp 1073751809"

key 0x02100027ac  session-ptr-vaddr 0x00cfb2a

Receive:

CallNH:desc_ptr:0xc3147, mode=1, mask=0x9c0

0x0c313c  4 : 0x23fffffc0000000c

0x0c313f  7 : 0xd033e59000002041

0x0c3140  8 : 0x421640000004e874

0x0c3141  9 : 0x12801005913a0800

Transmit/Age:

CallNH:desc_ptr:0xc3147, mode=1, mask=0x64c0

0x0c3139  1 : 0xd033e59000008241

0x0c313a  2 : 0x23fffffc0000000c

0x0c313d  5 : 0xd033e59000004041

0x0c3140  8 : 0x421640000004e874

0x0c3141  9 : 0x12801005913a0800

Arguments Data for Receive index 7:

JNH_KA_ARGS_DW0:

send_opaque_data = 0x00000000

aggregate_age_out_parcels = 0x00000000

max_age = 0x00000320

current_age = 0x0000006c

pkt_template_ptr = 0x000cfb22

JNH_KA_ARGS_DW1:

tx_vrf_valid = 0x00000001

tx_vrf = 0x00000000

tx_interval = 0x000000c8

tx_interval_curr = 0x0000005c

ts_present = 0x00000000

stats_present = 0x00000000

flags = 0x00000007

JNH_KA_ARGS_DW2:

opaque_data = 0x1db98c402231229f

JNH_KA_ARGS_DW3:

session_id = 0x00000001

pkt_data_paddr = 0x00000000

- % cprod -A fpc1 -c "show jnh inline-ka session 1 ppp 1073751809"

Entry Not found

%

Repeat the commands 3) and 4) multiple times (say about 10 times) with a time gap of 10 seconds between each iteration.

Run Commands 1) to 4) for few of the problematic pppoe sessions and share it with us.

Also, run the following commands for 2 iterations and share it with us. These are not specific to per subscriber.

- % cprod -A fpc1 -c "show jnh 0 exception terse"

- % cprod -A fpc1 -c "show jnh 1 exception terse"

- cprod -A fpc1 -c "show jnh 0 ucode-vars"

- cprod -A fpc1 -c "show jnh 1 ucode-vars"

- cprod -A fpc1 -c "show jnh inline-ka summary"

- cprod -A fpc1 -c "show jnh inline-ka mgmt"

- cprod -A fpc1 -c "show jnh inline-ka session 0 ppp global-stats"

- cprod -A fpc1 -c "show jnh inline-ka session 1 ppp global-stats"

- cprod -A fpc1 -c "show jnh inline-ka session 0 ppp host-outbound-info"

- cprod -A fpc1 -c "show jnh inline-ka session 1 ppp host-outbound-info"

- cprod -A fpc1 -c "show jnh inline-ka pfe 0 steering stats”

- cprod -A fpc1 -c "show jnh inline-ka pfe 1 steering stats”

- cprod -A fpc1 -c "show jnh inline-ka pfe 0 ae-info”

- cprod -A fpc1 -c "show jnh inline-ka pfe 1 ae-info”

- cprod -A fpc1 -c "show vbf pfe-events"

- cprod -A fpc1 -c "show jnh host-path-stats"

- cprod -A fpc1 -c "show vbf flow pppoe summary"